Initial ideation:

- Driving Mechanism:

- 2-wheel differential drive with caster wheel on front

- N20 Motor with encoder x2

- TB6612FNG due to its small size and efficiency

- Sensor and Microcontroller:

- TCRT5000, 8 line array

- Logic level shifter or just use two resistors

- ESP32

- Power:

- Buck Converter

- Normal current drawn around 700mA and peak current drawn can be around 3.5A, so capacity of 850mAh is enough.

- 2S LiPo, 850mAh of 10C+ rating

- Chassis Details:

- IR sensor is better run at 5V than 3.3V because then the sensor will work better making it more robust. And the working range for the IR sensor is between 1-15mm, 2.5mm being the optimum distance. But that is too short of a mechanical clearance so 5-6mm might be better.